OPOS DRIVER 1.13

Application Development Guide

For Ver. 1.13.0.1

Notes

- Unauthorized use of all or any part of this document is prohibited.

- The information in this document is subject to change without prior notice.

- This document has been created with full attention. If, however, you find an error or question, please contact us.

- We shall not be liable for any effect resulting from operation regardless of the above item 3.

- If you do not agree with the above terms, you are not permitted to use this driver.

Trademark

Microsoft, Windows, Windows Vista, Visual Studio, Visual Basic, and Visual C++ are registered trademarks of Microsoft Corporation in the United States and/or other countries. (Official name for Windows is Microsoft Windows Operating System.)

Company names and product names appearing on this document are trademarks and/or registered trademarks of respective companies.

Contents

1 Introduction	3

1.1 OPOS Driver	3

1.2 Operating Environment	3

1.3 Application Developer	3

1.4 OPOS Control	3

1.5 OPOS Driver Precautions	6

2 OPOS Printer Service Object	7

2.1 POS Printer Service Object and Method Relations	7

2.2 POS Printer Service Object and Property Relations	9

2.3 POS Printer Service Object and OPOS Escape Sequence Relations	16

2.4 POS Printer Service Object and Event Relations	17

2.5 POS Printer Service Object and Precautions	18

3 Cash Drawer Printer Service Object	20

3.1 Cash Drawer Service Object and Method Relations	20

3.2 Cash Drawer Service Object and Property Relations	22

3.3 Cash Drawer Service Object and Event Relations	24

3.4 Cash Drawer Service Object and Precautions	25

# Introduction

The OPOS Driver complies with OPOS1.13. This document provides the explanation of device (POS printer)-dependent specifications and precautions. When you want to develop applications by using this driver, refer to “UnifiedPOS Version 1.13” beforehand.

## OPOS Driver

This driver provides an open device driver architecture that can easily integrate the  POS printer and the cash drawer connected to it by the POS system based on Microsoft Windows Operating System. This driver has been developed for the following purpose.

- Definition of architecture for accessing the Win32-based POS printercash drawer connected

- to it .

- Definition of interface of POS printer supporting POS solution and cash drawer

- connected to it.

## Operating Environment

Object Operating System

This driver supports the following Windows operating systems.

- Windows XP

- Windows Vista

- Windows 7

- Windows 8

- Windows 10

From Windows Vista/7, operating system's registory structure is modified. Some applications that was programmed for Windows XP may not work correctly.

## Application Developer

- Application developer who wants to use this driver must be familiar with the following items.

- OPOS1.13 specifications

- General characteristics of   POS printer

- Terms and structure of ActiveX control and OLE automation

- ActiveX control container development environment such as Microsoft Visual Studio C# and        Microsoft Visual C++, etc.

## OPOS Control

The OPOS control, conforming to ActiveX control specifications, provides the application with

property, method, and/or event. It consists of control object (CO) for device class and service

object (SO). CO supplies application interface (API) and SO executes the API,

The OPOS software us implemented by the use of the layer shown in the following figure.

System

Registration

Mapping

Parameter

Control Object

Device class support

Device independent

Operating System/Driver

Printer           Cash Drawer

Intertace

ActiveX

Control

Int Interface

ActiveX

Controlrface

ActiveX

ControlInterface

ActiveX

Control Interface

ActiveX

ControlI Interface

ActiveX

Controlterface

ActiveX

Controlerface

ActiveX

Control

Intertace

ActiveX

Automation

Control Object

Device class support

Device independent

Method

OPOS

Control

ActiveX

Mapping

Parameter

Automation n Interface

ActiveX

Controlerface

ActiveX

Controlterface

ActiveX

Control

Method

Event

Method

Event

Method

Property

Opos

Control

Application

Service Object

Device dependent

Picture 1-1 OPOS software structure

Device Class

This is a category of the POS device sharing the set of property, method and event.

Control Object (CO)

Control Object provides the application with a set of property, method and event for each device

class. CO is a standard Active (OLE 32 bit) control and is not visible at the time of execution.

As a rule, this driver uses OPOS standard CO. (OPOS standard CO is installed automatically at

the installation of this driver.)

Service Object (SO)

Service Object is called by Control Object and executes the function specified by OPOS for each

device. It is implemented as automation server and provides the set of methods called by CO.

This driver provides the processing dependent on the   POS printer and cash drawer

connected to the printer.

## OPOS Driver Precautions

- In case of printer with USB interface, bidirectional communication which is registers to the Windows must be cancelled.

If you are using Windows XP, click "Start" button on the task bar and "Printers and Faxes".       Printers registered to Windows are shown in the opened window. Right-click the printer to be used by OPOS and click "Printer Properties" to show dialog described lator.

If you are using Windows Vista or Windows 7, click "Start" button on the task bar and "Devices and Printers". Printers registered to Windows are shown in the opened window. Right-click the printer to be used by OPOS and click "Printer Properties" to show dialog described lator.

Uncheck "Enable bidirectional support" check button in the "Ports" tab and click OK button.

In the following figure, the "Enable bidirectional support" checkbox is indexed by red arrow, and unchecked.

Picture 1-2 Disable Windows Printer Bidirectional Support

- Please note that proper operation is n guaranteed at the environment where OPOS driver by other company is installed.

- Also refer to the notes in each chapter.ot

# OPOS Printer Service Object

- For the details, method of use, etc. of property, method and event, refer to “UnifiedPOS Version 1.13”.

## POS Printer Service Object and Method Relations

Tabel 2-1 POS Printer Service Object and Method Relations

| Method | Status of support by the driver |  | Notes |
| --- | --- | --- | --- |
| Open | ✓ |  |  |
| Close | ✓ |  |  |
| ClaimDevice | ✓ |  |  |
| ReleaseDevice | ✓ |  |  |
| CheckHealth | ✓ | OPOS_CH_INTERNAL OPOS_CH_EXTERNAL |  |
| ClearInput |  | Not applied because no input device is used. | Not applicable with OPOS Printer |
| ClearOutput | ✓ |  | Data being transmitted cannot be stopped. |
| CompareFirmwareVersion |  | Not supported(OPOS_E_ILLEGAL) |  |
| DirectIO | ✓ |  |  |
| ResetStatistics |  | Not supported(OPOS_E_ILLEGAL) |  |
| RetrieveStatistics |  | Not supported(OPOS_E_ILLEGAL) |  |
| UpdateFirmware |  | Not supported(OPOS_E_ILLEGAL) |  |
| UpdateStatistics |  | Not supported(OPOS_E_ILLEGAL) |  |
| PrintNormal | ✓ |  |  |
| PrintTwoNormal |  | Not supported(OPOS_E_ILLEGAL) |  |
| PrintImmediate | ✓ |  |  |
| BeginInsertion |  | Not supported(OPOS_E_ILLEGAL) |  |
| EndInsertion |  | Not supported(OPOS_E_ILLEGAL) |  |
| BeginRemoval |  | Not supported(OPOS_E_ILLEGAL) |  |
| EndRemoval |  | Not supported(OPOS_E_ILLEGAL) |  |
| CutPaper | ✓ | Partial cut (1-99) ,Full cut (100) |  |
| RotatePrint | ✓ |  |  |
| PrintBarCode | ✓ | PTR_BCS_UPCA PTR_BCS_UPCE PTR_BCS_JAN8 PTR_BCS_EAN8 PTR_BCS_JAN13 PTR_BCS_EAN13 PTR_BCS_ITF PTR_BCS_Codabar PTR_BCS_Code39 PTR_BCS_Code93 PTR_BCS_Code128(Auto) PTR_BCS_Code128_Parsed( A,B,C) |  |
| PrintBitmap | ✓ |  |  |
| TransactionPrint | ✓ |  |  |
| ValidateData |  | Not supported (OPOS_E_ILLEGAL) |  |
| SetBitmap | ✓ | Raster bit image mode NV bit image mode | (Refer to Set Bitmap Method/ESC¦#B Escape Sequence in 2.5. POS Printer Service Object and Precautions.) |
| SetLogo | ✓ |  |  |
| ChangePrintSide |  | Not supported (OPOS_E_ILLEGAL) |  |
| MarkFeed |  | Not supported (OPOS_E_ILLEGAL) |  |
| ClearPrintArea | ✓ |  |  |
| PageModePrint | ✓ |  |  |
| PrintMemoryBitmap |  | Not supported (OPOS_E_ILLEGAL) |  |
| DrawRuledLine |  | Not supported (OPOS_E_ILLEGAL) |  |

## POS Printer Service Object and Property Relations

Tabel 2-2 POS Printer Service Object and Property Relations

| Property | Attri bute | Status of support by the driver |  | Notes |  |
| --- | --- | --- | --- | --- | --- |
| BinaryConversion | R/W | ✓ | OPOS_BC_NONE (Default) OPOS_BC_NIBBLE OPOS_BC_DECIMAL | (Refer to Binary Conversion in 2.5.POS Printer Service Object and Precautions.) |  |
| CapCompareFirmwareVersion | R |  | FALSE |  |  |
| CapPowerReporting | R |  | FALSE |  |  |
| CapUpdateFirmware | R |  | FALSE |  |  |
| CapStatisticsReporting | R |  | FALSE |  |  |
| CheckHealthText | R | ✓ | External HCheck: Successful |  |  |
| Claimed | R | ✓ | ClaimeDevice → True ReleaseDevice → False |  |  |
| DeviceEnabled | R/W | ✓ | TRUE FALSE |  |  |
| FreezeEvents | R/W | ✓ | TRUE： Event notification disabled FALSE: Event notification enabled |  |  |
| OpenResult | R | ✓ | In other than OPOS_SUCCESS, refer to OpenResult properties in “UnifiedPOS Version 1.13”. |  |  |
| OutputID | R | ✓ | Increment (+1) | (Refer to OutPutID property/ OutputCompleteEvent event in 2.5.POS Printer Service Object and Precautions.) |  |
| PowerNotify | R/W |  | Not supported |  |  |
| PowerState | R |  | Not supported |  |  |
| ResultCode | R | ✓ | In other than OPOS_SUCCESS, refer to ResulCode properties in “UnifiedPOS Version 1.13”. | (Refer to ResultCode/ ResultCodeExtended Property in |  |
| ResultCodeExtended | R | ✓ |  | (Refer to ResultCode/ ResultCodeExtended Property in |  |
| State | R | ✓ | In other than OPOS_S_IDLE, refer to State properties in “UnifiedPOS Version 1.13”. |  |  |
| ControlObjectDescription | R | ✓ | OPOS POSPrinter Control 1.13.001 [Public, by CRM/RCS-Dayton] |  |  |
| ControlObjectVersion | R | ✓ | 1013001 |  |  |
| ServiceObjectDescription | R | ✓ | OPOS POSPrinter Service Object 1.13.001 |  |  |
| ServiceObjectVersion | R | ✓ | 1011001 |  |  |
| DeviceDescription | R | ✓ | High Performance Printer |  |  |
| DeviceName | R | ✓ | Printer |  |  |
| CapCharacterSet | R | ✓ | PTR_CCS_ASCII |  |  |
| CapConcurrentJrnRec | R |  | Not supported |  |  |
| CapConcurrentPageMode | R |  | Not supported |  |  |
| CapConcurrentJrnSlp | R |  | Not supported |  |  |
| CapConcurrentRecSlp | R |  | Not supported |  |  |
| CapCoverSensor | R | ✓ | TRUE: With cover open sensor |  |  |
| CapTransaction | R | ✓ | TRUE: Batch processing of characters is possible. |  |  |
| CapJrnPresent | R |  | Not supported |  |  |
| CapJrn2Color | R |  | Not supported |  |  |
| CapJrnBold | R |  | Not supported |  |  |
| CapJrnDhigh | R |  | Not supported |  |  |
| CapJrnDwide | R |  | Not supported |  |  |
| CapJrnDwideDhigh | R |  | Not supported |  |  |
| CapJrnEmptySensor | R |  | Not supported |  |  |
| CapJrnItalic | R |  | Not supported |  |  |
| CapJrnNearEndSensor | R |  | Not supported |  |  |
| CapJrnUnderline | R |  | Not supported |  |  |
| CapJrnCartridgeSensor | R |  | Not supported |  |  |
| CapJrnColor | R |  | Not supported |  |  |
| CapRecPresent | R | ✓ | TRUE： Receipt is possible. |  |  |
| CapRec2Color | R |  | Not supported |  |  |
| CapRecBarCode | R | ✓ | TRUE: BarCode printing is possible. |  |  |
| CapRecBitmap | R | ✓ | TRUE: Bitmap printing is possible. |  |  |
| CapRecBold | R | ✓ | TRUE: Bold attribute setting is possible. |  |  |
| CapRecDhigh | R | ✓ | TRUE: Double height attribute setting is possible. |  |  |
| CapRecDwide | R | ✓ | TRUE: Double width setting is possible. |  |  |
| CapRecDwideDhigh | R | ✓ | TRUE: Quadruple attribute setting is possible. |  |  |
| CapRecEmptySensor | R | ✓ | TRUE: Paper-Out sensor is active. FALSE: Paper-Out sensor is nonactive. |  |  |
| CapRecItalic | R |  | Not supported |  |  |
| CapRecLeft90 | R | ✓ | TRUE: Rotate Left 90 is possible. |  |  |
| CapRecNearEndSensor | R | ✓ | TRUE: With Paper Near-End sensor |  |  |
| CapRecPapercut | R | ✓ | Only partial cut (90) is supported. |  |  |
| CapRecRight90 | R | ✓ | TRUE: Rotate Right 90 is possible. |  |  |
| CapRecRotate180 | R | ✓ | TRUE: Rotate 180 is possible. |  |  |
| CapRecStamp | R |  | Not supported |  |  |
| CapRecUnderline | R | ✓ | TRUE: Underline attribute setting is possible. |  |  |
| CapRecCartridgeSensor | R | ✓ | PTR_CART_OK |  |  |
| CapRecColor | R | ✓ | PTR_COLOR_PRIMARY |  |  |
| CapRecMarkFeed | R |  | Not supported |  |  |
| CapRecPageMode | R | ✓ | TRUE: PageMode is possible. |  |  |
| CapRecRuledLine | R |  | Not supported |  |  |
| CapSlpPresent | R |  | Not supported |  |  |
| CapSlpFullslip | R |  | Not supported |  |  |
| CapSlp2Color | R |  | Not supported |  |  |
| CapSlpBarCode | R |  | Not supported |  |  |
| CapSlpBitmap | R |  | Not supported |  |  |
| CapSlpBold | R |  | Not supported |  |  |
| CapSlpDhigh | R |  | Not supported |  |  |
| CapSlpDwide | R |  | Not supported |  |  |
| CapSlpDwideDhigh | R |  | Not supported |  |  |
| CapSlpEmptySensor | R |  | Not supported |  |  |
| CapSlpItalic | R |  | Not supported |  |  |
| CapSlpLeft90 | R |  | Not supported |  |  |
| CapSlpRight90 | R |  | Not supported |  |  |
| CapSlpRotate180 | R |  | Not supported |  |  |
| CapSlpUnderline | R |  | Not supported |  |  |
| CapSlpBothSidesPrint | R |  | Not supported |  |  |
| CapSlpCartridgeSensor | R |  | Not supported |  |  |
| CapSlpColor | R |  | Not supported |  |  |
| CapSlpPageMode | R |  | Not supported |  |  |
| CapSlpRuledLine | R |  | Not supported |  |  |
| AsyncMode | R/W | ✓ | TRUE/FALSE | (Refer to AsyncMode Property in 2.5.POS Printer Service Object and Precautions.) |  |
| CartridgeNotify | R/W |  | Not supported |  |  |
| CharacterSet | R/W | ✓ | 437: English version |  |  |
| CharacterSetList | R | ✓ | 437: OEM United States 737: Greek (DOS) 850: Western European 852: Central European (DOS) 855: OEM Cyrillic 857: Turkish (DOS) 858: OEM Multilingual Latin I 860: Portuguese (DOS) 862: Hebrew (DOS) 864: Arabic 865: Nordic (DOS) 874: Thai (Windows) 932: Japanese (Shift-JIS) 936: Chinese Simplified (GB2312) 949: Korean 950: Chinese Traditional (Big5) 1047: IBM Latin-1 1250: Central European (Windows) 1255: Hebrew (Windows) 1256: Arabic (Windows) 28591: Western European (ISO) |  |  |
| CoverOpen | R | ✓ | TRUE: POS printer cover is open. FALSE: POS printer cover is closed. |  |  |
| ErrorLevel | R | ✓ | PTR_EL_NONE PTR_EL_RECOVERABLE PTR_EL_FATAL | (Refer to ErrorLevel Property in 2.5.POS Printer Service Object and Precautions.) |  |
| ErrorStation | R | ✓ | PTR_S_RECEIPT |  |  |
| ErrorString | R | ✓ | Description on present error is retained. |  |  |
| FontTypefaceList | R | ✓ | Font A, B |  |  |
| FlagWhenIdle | R/W | ✓ |  |  |  |
| MapMode | R/W | ✓ | PTR_MM_DOTS(Default) PTR_MM_TWIPS PTR_MM_ENGLISH PTR_MM_METRIC |  |  |
| PageModeArea | R |  | 576,936 (PTR_MM_DOTS) |  |  |
| PageModeDescriptor | R |  | PTR_PM_BITMAP PTR_PM_BARCODE PTR_PM_BM_ROTATE PTR_PM_BC_ROTATE |  |  |
| PageModeHorizontalPosition | R/W |  | 0 |  |  |
| PageModePrintArea | R/W |  | 0,0, 576,936 |  |  |
| PageModePrintDirection | R/W |  | PTR_PD_LEFT_TO_RIGHT(Default) PTR_PD_BOTTOM_TO_TOP PTR_PD_RIGHT_TO_LEFT PTR_PD_TOP_TO_BOTTOM |  |  |
| PageModeStation | R/W |  | PTR_S_RECEIPT |  |  |
| PageModeVerticalPosition | R/W |  | 0 |  |  |
| RotateSpecial | R/W |  | PTR_RP_NORMAL(Default) PTR_RP_RIGHT90 PTR_RP_LEFT90 PTR_RP_ROTATE180 |  |  |
| JrnLineChars | R/W |  | Not supported |  |  |
| JrnLineCharsList | R |  | Not supported |  |  |
| JrnLineHeight | R/W |  | Not supported |  |  |
| JrnLineSpacing | R/W |  | Not supported |  |  |
| JrnLineWidth | R |  | Not supported |  |  |
| JrnLetterQuality | R/W |  | Not supported |  |  |
| JrnEmpty | R |  | Not supported |  |  |
| JrnNearEnd | R |  | Not supported |  |  |
| JrnCartridgeState | R |  | Not supported |  |  |
| JrnCurrentCartridge | R/W |  | Not supported |  |  |
| RecLineChars | R/W | ✓ | 48 (Default) OPOS_SUCCESS (Normal setting) OPOS_E_ILLEGAL (Illegal line character width is specified.) | According to the value, the Font size is changed. Value should be chosen from RecLineCharsList property. (Refer to RecLineChars/RecLineCharsList Property in 2.5 POS Printer Service Object and Precautions.) |  |
| RecLineCharsList | R | ✓ | 48,64 | (Refer to RecLineChars/RecLineCharsList Property in 2.5 POS Printer Service Object and Precautions.) |  |
| RecLineHeight | R/W | ✓ | 24 | The value is changed according to the RecLineChars property. |  |
| RecLineSpacing | R/W | ✓ | 30 | If the value is smaller than RecLineHeight, the value is changed according to the RecLineHeight property. |  |
| RecLineWidth | R | ✓ | 512 |  |  |
| RecLetterQuality | R/W | ✓ |  |  |  |
| RecEmpty | R | ✓ | TRUE: Receipt paper out FALSE: With receipt paper |  |  |
| RecNearEnd | R | ✓ | TRUE: Receipt paper near end FALSE: With receipt paper |  |  |
| RecSidewaysMaxLines | R | ✓ |  |  |  |
| RecSidewaysMaxChars | R | ✓ |  |  |  |
| RecLinesToPaperCut | R | ✓ | 5 |  |  |
| RecBarCodeRotationList | R | ✓ | 0,R90,L90,180 |  |  |
| RecCartridgeState | R |  | 0 |  |  |
| RecCurrentCartridge | R/W | ✓ | PTR_COLOR_PRIMARY |  |  |
| RecBitmapRotationList | R | ✓ | 0,R90,L90,180 |  |  |
| SlpLineChars | R/W |  | Not supported |  |  |
| SlpLineCharsList | R |  | Not supported |  |  |
| SlpLineHeight | R/W |  | Not supported |  |  |
| SlpLineSpacing | R/W |  | Not supported |  |  |
| SlpLineWidth | R |  | Not supported |  |  |
| SlpLetterQuality | R/W |  | Not supported |  |  |
| SlpEmpty | R |  | Not supported |  |  |
| SlpNearEnd | R |  | Not supported |  |  |
| SlpSidewaysMaxLines | R |  | Not supported |  |  |
| SlpSidewaysMaxChars | R |  | Not supported |  |  |
| SlpMaxLines | R |  | Not supported |  |  |
| SlpLinesNearEndToEnd | R |  | Not supported |  |  |
| SlpBarCodeRotationList | R |  | Not supported |  |  |
| SlpPrintSide | R |  | Not supported |  |  |
| SlpCartridgeState | R |  | Not supported |  |  |
| SlpCurrentCartridge | R/W |  | Not supported |  |  |

## POS Printer Service Object and OPOS Escape Sequence Relations

Tabel 2-3 POS Printer Service Object and OPOS Escape Sequence Relations

| Escape Sequence |  | Status of support by the driver |  | Notes |
| --- | --- | --- | --- | --- |
| Paper cut | ESC¦[#]P | ✓ | 0-100 | Partial cut (0-99) ,Full cut (100) |
| Feed and paper cut | ESC¦[#]fP | ✓ | 0-100 |  |
| Feed, cut, and stamp print | ESC¦[#]sP |  | Not supported |  |
| Bitmap print | ESC¦#B | ✓ | 1-20 Specifies Bitmap number registered by the Set Bit map method. Printing is carried out in raster bit image or NV bit image mode. | After Bitmap printing, print position returns to the initial state (left-justified). (Refer to Set Bitmap Method/ESC¦#B Escape Sequence in 2.5. POS Printer Service Object and Precautions.) |
| Top logo print | ESC¦tL | ✓ |  |  |
| Bottom logo print | ESC¦bL | ✓ |  |  |
| Stamp print | ESC¦sL |  | Not supported |  |
| Multi-line feed | ESC¦[#]lF | ✓ |  |  |
| Unit feed | ESC¦[#]uF | ✓ |  |  |
| Reverse feed | ESC¦[#]rF |  | Not supported |  |
| Embedded data | ESC¦#E | ✓ |  |  |
| Barcode | ESC¦#R | ✓ |  |  |
| Ruled-line | ESC¦#dL |  | Not supported |  |
| Font type | ESC¦#fT | ✓ |  |  |
| Bold | ESC¦[!]bC | ✓ |  | If "!" is specified, then bold is disabled. |
| Underline | ESC¦[!][#]uC | ✓ |  | If "!" is specified, then underline is disabled. |
| Italic | ESC¦[!]iC |  | Not supported |  |
| Custom color | ESC¦[#]rC |  | Not supported |  |
| Reverse character | ESC¦[!]rvC | ✓ |  | If "!" is specified, then reverse character is disabled. |
| Shaded character | ESC¦[#]sC |  | Not supported |  |
| Standard | ESC¦1C | ✓ |  |  |
| Double width | ESC¦2C | ✓ |  |  |
| Double height | ESC¦3C | ✓ |  |  |
| Quadruple | ESC¦4C | ✓ |  |  |
| Horizontal magnification | ESC¦[#]hC | ✓ | 1, 2, 4, 8 |  |
| Vertical magnification | ESC¦[#]vC | ✓ | 1, 2, 4, 8 |  |
| Color specification | ESC¦[#]fC |  | Not supported |  |
| Centering | ESC¦cA | ✓ |  |  |
| Right adjustment | ESC¦rA | ✓ |  |  |
| Left adjustment | ESC¦lA | ✓ |  |  |
| Normal | ESC¦N | ✓ |  |  |
| Strike-through | ESC¦[!][#]stC |  | Not supported |  |
| Subscript | ESC¦[!]tbC |  | Not supported |  |
| Superscript | ESC¦[!]tpC |  | Not supported |  |

## POS Printer Service Object and Event Relations

Tabel 2-4 POS Printer Service Object and Event Relations

| Event | Status of support by the driver |  | Notes |
| --- | --- | --- | --- |
| DataEvent |  | Not supported | Not applicable with OPOSPrinter |
| DirectIOEvent |  | Not supported | Not applicable with OPOSPrinter |
| ErrorEvent | ✓ |  | (Refer to ErrorEvent in 2.5 POS Printer Service Object and Precautions.) |
| OutputCompleteEvent | ✓ |  | (Refer to OutPutID property/Output Complete Event event in 2.5 POS Printer Service Object and Precautions.) |
| StatusUpdateEvent | ✓ |  |  |

## POS Printer Service Object and Precautions

RotatePrint Method

Printing size dynamically changes by printing data within the value of RecSidewaysMaxChars.

SetBitmap Method/ESC|#B Escape Sequence

Bitmap is registered by specifying Bitmap number (1 - 20) using the SetBitmap method and

bitmap printing is carried out by specifying the registered number in the ESC|#B escape

sequence. This driver has two modes, raster bit image mode and NV bit image mode.

Specification of each mode is as shown below.

- Raster bit image mode
SO stores own bitmap using the SetBitmap method. When SO is closed, the stored bitmap is cleared. Therefore, it is necessary to register bitmap each time SO is opened. In this mode, bitmap is memorized including printing position designated at the time of registration.

- NV bit image mode
By using the SetBitmap method, SO writes bitmap to the nonvolatile memory (Flash ROM) of the printer. 
Therefore, it is not necessary to register bitmap each time SO is opened. 
In this mode, the printing position designated at the time of registration is not stored and printing starts from the left end.

AsyncMode Property

- TRUE

Output method (PrintNormal, CutPaper, RotatePrint, PrintBarCode, PrintBitmap, PageModePrint and TransactionPrint) is executed asynchronously.

- FALSE

Output method is printed synchronously. When error occurs at the asynchronous output,error event is reported at about 0.5-s intervals.

ErrorLevel Property

- PTR_EL_NONE

Indicated when error is not detected.

- PTR_EL_RECOVERABLE

Indicated as recoverable error when Paper Out/Cover Open.

- PTR_EL_FATAL

Indicated as irrecoverable error at Auto Cutter error happened.

BinaryConversion Property

Installation example (C#) using the BinaryConversion property and PrintNormal method is described.

- Opos_BC_NONE

Character is stored with one byte without data conversion.

- Opos_BC_NIBBLE

Each byte is converted to two characters.

- -OPOS_BC_DECIMAL

Each byte is converted to three characters. To create three ASCII characters from each byte,

You can use fuction: String.Format("0:d3",Byte) to implement this convert.

ErrorEvent

Notifies the application that a POS Printer error has been detected and that a

suitable response by the application is necessary to process the error condition.

In case of ErrorCode, refer to Error Codes Property in  “UnifiedPOS Version 1.13”.

If ErrorCode is E_EXTENDED, then ErrorCodeExtendedhas one of the

following values:

- EPTR_COVER_OPEN

- Indicated the printer cover is open.

- EPTR_REC_EMPTY

Indicated the receipt station is out of paper.

ErrorLocus value is EL_OUTPUT, ErrorResponse value is ER_RETRY.

OutPutID Property/ OutputCompleteEvent Event

Identifier of asynchronous output started just before is retained and incremented by +1 with the

following output.

RecLineChars/ RecLineCharsList Property

The number of one-byte characters of the font printed in one line on the paper is specified.

- RecLineChars property: Specifies the maximum number of digits of FontA corresponding to the

specified paper size.

- RecLineCharsList property: Lists the maximum number of digits of FontA, FontB corresponding to the specified paper size.

ResultCode/ ResultCodeExtended Property

When the value of the ResultCode property is other than OPOS_SUCCESS, refer to the

ResultCode property in  “UnifiedPOS Version 1.13”

In case of OPOS_E_Extended, refer to ResultCodeExtended Property in  “UnifiedPOS Version 1.13”

# Cash Drawer Printer Service Object

For the details, method of use, etc. of property, method and event, refer to “UnifiedPOS Version 1.13”.

## Cash Drawer Service Object and Method Relations

Tabel 3-5 Cash Drawer Service Object and Method Relations

| Method | Status of support by the driver |  | Notes |
| --- | --- | --- | --- |
| Open | ✓ |  |  |
| Close | ✓ |  |  |
| ClaimDevice | ✓ |  |  |
| ReleaseDevice | ✓ |  |  |
| CheckHealth | ✓ | OPOS_CH_INTERNAL OPOS_CH_EXTERNAL |  |
| ClearInput |  | Not supported(OPOS_E_ILLEGAL) | Not applicable with OPOS Drawer. |
| ClearOutput |  | Not supported(OPOS_E_ILLEGAL) | Not applicable with OPOS Drawer. |
| CompareFirmwareVersion |  | Not supported |  |
| DirectIO |  | Not supported(OPOS_E_ILLEGAL) |  |
| ResetStatistics |  | Not supported(OPOS_E_ILLEGAL) |  |
| RetrieveStatistics |  | Not supported(OPOS_E_ILLEGAL) |  |
| UpdateFirmware |  | Not supported |  |
| UpdateStatistics |  | Not supported(OPOS_E_ILLEGAL) |  |
| OpenDrawer | ✓ |  |  |
| WaitForDrawerClose | ✓ |  |  |

## Cash Drawer Service Object and Property Relations

Tabel 3-6 Cash Drawer Service Object and Property Relations

| Property | Attri bute | Status of support by the driver |  | Notes |
| --- | --- | --- | --- | --- |
| AutoDisable | R/w |  |  | Not applicable with OPOS Drawer. |
| BinaryConversion | R/W |  |  | Not applicable with OPOS Drawer. |
| CapCompareFirmwareVersion | R |  | FALSE |  |
| CapPowerReporting | R | ✓ | OPOS_PR_NONE | (Refer to CapPowerReporting/ Powermotify/ PowerState Property in 3.4.Cash Drawer Service Object and Precautions.) |
| CapUpdateFirmware | R |  | FALSE |  |
| CapStatisticsReporting | R |  | FALSE |  |
| CheckHealthText | R | ✓ | External HCheck: Successful |  |
| Claimed | R | ✓ | TRUE/FALSE |  |
| DeviceEnabled | R/W | ✓ | TRUE/FALSE |  |
| FreezeEvents | R/W | ✓ | TRUE： Event notification disabled FALSE: Event notification enabled |  |
| OpenResult | R | ✓ | OPOS_SUCCESS |  |
| OutputID | R |  |  | Not applicable with OPOS Drawer. |
| PowerNotify | R/W |  | FALSE |  |
| PowerState | R |  | FALSE |  |
| ResultCode | R | ✓ |  |  |
| ResultCodeExtended | R | ✓ |  |  |
| State | R | ✓ |  |  |
| ControlObjectDescription | R | ✓ | OPOS CashDrawerr Control 1.13.001 [Public, by CRM/RCS-Dayton] |  |
| ControlObjectVersion | R | ✓ |  |  |
| ServiceObjectDescription | R | ✓ | POS Printer OPOS Service Object 1.13.001 UPOS 1.13, Copyright (C) 2013- CORP |  |
| ServiceObjectVersion | R | ✓ | 1013001 |  |
| DeviceDescription | R | ✓ | POS CashDrawer |  |
| DeviceName | R | ✓ | Printer |  |
| CapStatus | R | ✓ |  |  |
| CapStatusMultiDrawerDetect | R | ✓ | FALSE |  |
| DrawerOpened | R | ✓ |  |  |

## Cash Drawer Service Object and Event Relations

Tabel 3-7 Cash Drawer Service Object and Event Relations

| Event | Status of support by the driver |  | Notes |
| --- | --- | --- | --- |
| DataEvent |  | Not supported | Not applicable with OPOSDrawer |
| DirectIOEvent |  | Not supported | Not applicable with OPOSDrawer |
| ErrorEvent |  | Not supported | Not applicable with OPOSDrawer |
| OutputCompleteEvent |  | Not supported | Not applicable with OPOSDrawer |
| StatusUpdateEvent | ✓ |  |  |

## Cash Drawer Service Object and Precautions

CapPowerReporting/ Powermotify/ PowerState Property

This service object supports only cash drawer connected to the drawer kick connector at the back

of the printer. Therefore the value of the CapPowerReporting property shall be

“OPOS_PR_NONE”.

CapStatusMultiDrawerDetect Property

Two cash drawers can be connected by using Y-Shape cable to drawer kick connector at this

service object but individual detection of cash drawer open/close situation is not possible. For

this reason, CapStatusMultiDrawerDetect Property value becomes “FALSE”.

When CapStatusMultiDrawerDetect Property is “FALSE”, only the following status is informed.

a) All cash drawers are closed.

b) One or more cash drawer(s) is open.

In addition, each [Pin 3 Signal at Drawer Open Status] should be set in the same way when

registering multiple numbers of cash drawers from [OPOS Driver Setting Tool].

Picture Contents

Picture 1-1  OPOS software structure	5

Picture 1-2  Disable Windows Printer Bidirectional Support	6

Table Contents

Tabel 2-1  POS Printer Service Object and Method Relations	7

Tabel 2-2  POS Printer Service Object and Property Relations	9

Tabel 2-3  POS Printer Service Object and OPOS Escape Sequence Relations	16

Tabel 2-4  POS Printer Service Object and Event Relations	17

Tabel 3-1  Cash Drawer Service Object and Method Relations	20

Tabel 3-2  Cash Drawer Service Object and Property Relations	22

Tabel 3-3  Cash Drawer Service Object and Event Relations	24